Parameters, sources & methods for personal CO2 equivalent (CO2e) estimates and global benchmarks

A survey-driven model for annual lifestyle CO₂e emissions, for respondents in BC, AB, QC, ON (Canada) and CO, MI, NY, WA (US).

# Diet

O’Malley, K., Willits-Smith, A. and Rose, D., 2023. Popular diets as selected by adults in the United States show wide variation in carbon footprints and diet quality. The American Journal of Clinical Nutrition, 117(4), pp.701-708.

| Diet type | t CO2e | notes |
| --- | --- | --- |
| Omnivore | 1.6279 | O’Malley reports 2.23 kg CO2e /1000 kcal for omnivore diet. We assume 2,000 kcal/day and obtain an annual estimate as: 2.23 x 2 x 365 |
| Flexitarian | 1.23735 | Mean of omnivore and vegetarian value |
| Vegetarian | 0.8468 | As for omnivore, using O’Malley’s value for vegetarian diet (1.16 kg CO2e) |
| Vegan | 0.5037 | As for omnivore, using O’Malley’s value for vegan diet (0.69 kg CO2e) |

# Air travel

Flight emissions are calculated via Atmosfair: https://co2offset.atmosfair.de/co2offset#/ [last accessed: 02.04.2026].

Flight distance categories (short, medium, long-haul) are adopted from the U.S. Environmental Protection Agency:

U.S. Environmental Protection Agency (2025). GHG Emission Factors Hub. Retrieved from https://www.epa.gov/climateleadership/ghg-emission-factors-hub.

Short haul < 300 miles

Medium haul >300 miles & < 2,300 miles

Long haul > 2,300 miles

Next, an LLM-assisted search using Claude 4.5 (Sonnet version) was performed to identify 3 to 5 popular travel destinations in each category from each of the 6 largest cities in our sample (New York City, Toronto, Montreal, Seattle, Detroit, Vancouver). These are shown the Appendix below.

For each of these flights, a search on the travel booking site Kayak (https://www.ca.kayak.com/) was performed to identify first flight listed in Kayak’s results. The CO2e emissions for this “best” itinerary was then calculated using Atmosfair’s offset calculator (https://www.atmosfair.de/en/offset/flight/). Mean CO2e emissions per distance category were calculated for economy class flights, with upstream fuel extraction and processing costs added based on the GREET/RESNET-consistent aviation value (~14.8 kg CO₂e/MMBtu HHV ≈ +20.5% on CO₂). The climate impact of contrails and ozone formation, as measured by Atmosfair were then added. By this method, CO2e values for long-haul, medium, and short flights are estimated as 2.536, 0.7701, and 0.2205 tons respectively.

# Residential energy use

Survey inputs: jurisdiction, dwelling vintage, dwelling type, heated floor area, space-heating system, water-heating system, air conditioning (yes/no), household size, and recent window/insulation upgrade.

## Model

CO₂e (kg/yr) = Space heating + Water heating + Air conditioning + Other electricity

- Space heating = Demand D (GJ/m²) × heated area (m²) × envelope factor × retrofit factor ÷ system efficiency × fuel factor

- Water heating = useful hot-water demand U (GJ/household, by dwelling type) ÷ system efficiency × fuel factor

- Air conditioning = cooling intensity (GJ/m²) × floor area × envelope factor × retrofit factor × grid factor (if AC = yes)

- Other electricity = appliances/lighting/plug load (GJ/household, by household size) × grid factor

Floor area is an explicit input, so space heating and cooling are fully per-m²; the calculator also reports a per-m² and per-ft² intensity so dwellings of different sizes can be compared directly. Heating and water heating start from a fuel-neutral energy demand divided by the reported equipment’s efficiency.

Dwelling type drives an envelope factor (detached 1.00 / attached 0.85 / apartment 0.70) on per-m² heating and cooling, plus a load factor on water heating. Household size drives the occupant loads — other electricity (and, optionally, water heating).

## Space-heating useful heat demand, D (GJ per m² per year)

| Vintage | QC | ON | AB | BC | CO | MI | NY | WA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pre-1960 | 0.85 | 0.85 | 0.90 | 0.55 | 0.31 | 0.46 | 0.33 | 0.28 |
| 1960–1983 | 0.65 | 0.55 | 0.65 | 0.40 | 0.28 | 0.36 | 0.32 | 0.25 |
| 1984–1999 | 0.45 | 0.40 | 0.45 | 0.30 | 0.25 | 0.29 | 0.26 | 0.23 |
| 2000+ | 0.34 | 0.33 | 0.36 | 0.22 | 0.20 | 0.27 | 0.22 | 0.18 |

Sources — Canada (useful heat demand by vintage, single detached): NRCan CEUD Residential Table 33, 2023: QC · ON · AB · BC. US (gas-heated-home delivered intensity × 0.85 fleet-average AFUE, state × vintage): EIA RECS 2020 microdata.

### Furnace efficiency (AFUE):

D is useful heat demand — the heat the dwelling needs, independent of equipment — so the calculator divides it by whatever system a respondent reports. This value is reached differently for Canada and the US:

- Canada — no adjustment. NRCan Table 33 publishes “Gross Output Thermal Requirements per m²,” already the heat the equipment must deliver.

- US — adjustment needed. RECS measures the gas a home consumes, not the heat delivered, so consumed gas is multiplied by the furnace’s seasonal efficiency (AFUE), the fraction of fuel that becomes useful heat, to reach useful heat. The US gas-furnace stock averages ~0.85 (federal minimum 0.80 AFUE since 2015; condensing units at 0.90–0.97 are only ~40–45% of the installed stock), to derive the US demand.

Furnace efficiency is province/state-specific, taken from the actual stock mix. NRCan reports the gas-furnace stock split across efficiency tiers (Table 21) and the tier efficiencies (Table 26: Medium 80%, High/condensing 90%). The stock-weighted fleet average for 2023:

| Parameter | ON | QC | AB | BC | CO | MI | NY | WA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gas-furnace fleet efficiency | 0.89 | 0.88 | 0.87 | 0.87 | 0.85 | 0.85 | 0.85 | 0.85 |

Canada’s gas fleet is more efficient than the US: Ontario’s gas stock is ~87% high-efficiency (condensing), versus ~40–45% in the US; Alberta and BC retain more medium-efficiency units (~0.87). US states use the ~0.85 national value as state-specific data is not available.

Sources: NRCan CEUD Table 21, Heating System Stock by Type — ON · QC · AB · BC; NRCan CEUD Table 26, Heating System Stock Efficiencies; US fleet — ENERGY STAR furnace criteria and US DOE residential furnace standards.

## Dwelling-type factors

| Dwelling type | Envelope factor (heating & cooling) | Load factor — water heating |
| --- | --- | --- |
| detached | 1.00 | 1.13 |
| attached | 0.85 | 1.03 |
| apartment | 0.70 | 0.77 |

The envelope factor (per-m²) comes from NRCan’s building-type thermal-demand ratios (Table 33). The water-heating load factor comes from RECS 2020 by dwelling type (19.2 / 17.4 / 13.1 GJ/hh for detached / attached / apartment), normalized so the all-types average reproduces the provincial value.

Sources: NRCan CEUD Table 33 (envelope; links in §1); EIA RECS 2020 microdata (load factors by dwelling type).

# Renovation adjustment — windows / insulation upgrades

The survey asks: “Have windows or insulation been significantly upgraded since the home was built? (yes / no / unsure).” A “yes” multiplies both the space-heating demand D and the cooling intensity by 0.85 (a 15% reduction); “no” and “unsure” leave them unchanged (factor 1.00), so an upgrade is never assumed.

The 15% value is a conservative central estimate within the 10–25% range reported for envelope retrofits. The US Weatherization Assistance Program reports average whole-home energy savings above 20% (a bundle that also includes air-sealing and some system measures); NRCan EnerGuide retrofit evaluations show typical insulation and air-sealing jobs improving home energy performance by roughly 10–25%, with deep retrofits exceeding 50%; individual measures are smaller (attic/wall insulation ~10–20%, window replacement ~5–10% of heating).

Sources: US DOE Weatherization Assistance Program — Whole-House Weatherization; New York State Weatherization Assistance Program (~20% average savings); NRCan EnerGuide home energy evaluations.

## Heating-system efficiency / COP and fuel

| System | Fuel | Efficiency / COP |
| --- | --- | --- |
| Natural gas furnace | Natural gas | 0.85–0.89 (province/state-specific — see §1) |
| Heating oil / propane furnace | Oil/propane | 0.84 (mean of oil 0.83, propane 0.85) |
| Electric baseboard / resistance | Electricity | 1.00 |
| Electric heat pump (air-source) | Electricity | 1.9 (Canada) / 2.0 (US) — seasonal COP; see below |
| Wood stove | Wood | 0.65 |

Heat-pump seasonal COP — Canada 1.9, US 2.0, each from its own national source:

| Jurisdiction group | Seasonal COP | Source |
| --- | --- | --- |
| Canada (QC, ON, AB, BC) | 1.9 | NRCan CEUD Table 26 — national stock-average, identical across provinces |
| US (CO, MI, NY, WA) | 2.0 | US federal-minimum HSPF 6.8 ÷ 3.412 (EIA characterizes heat pumps by HSPF; RECS reports no COP) |

COP values are stock-level figures that include electric-resistance backup on the coldest days, and hence are conservative.

Furnace AFUEs are mid-efficiency (condensing gas ≈ 0.95); resistance heating is 1.0 by definition.

Sources: heat-pump COP — Canada 1.9 NRCan CEUD Table 26, Heating System Stock Efficiencies (identical across ON/QC/AB/BC); US has no published stock COP — EIA Annual Energy Outlook, Residential Demand Module (HSPF assumptions), US DOE air-source heat pumps; modern-unit performance — NEEP ccASHP specification, CanmetENERGY field assessment; furnace AFUE — ENERGY STAR furnaces.

## Water heating — useful hot-water demand U (GJ per household per year), by dwelling type

Final energy = U ÷ water-heater efficiency.

| Dwelling type | QC | ON | AB | BC | CO | MI | NY | WA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| detached | 12.4 | 11.2 | 19.8 | 15.8 | 12.8 | 11.6 | 12.0 | 14.7 |
| attached | 11.3 | 10.2 | 18.0 | 14.4 | 11.6 | 10.6 | 10.9 | 13.4 |
| apartment | 8.5 | 7.6 | 13.5 | 10.8 | 8.7 | 7.9 | 8.2 | 10.0 |

Water-heater efficiency by system: electric resistance 0.90, natural gas 0.60, heating oil / propane 0.59 (mean of oil 0.58 & propane 0.60), heat-pump water heater 2.5 (COP).

Sources — Canada (water-heating energy per household): NRCan CEUD Residential Table 2 (end-use), 2023: QC · ON · AB · BC. US and dwelling-type variation: EIA RECS 2020 microdata. Water-heater efficiencies (UEF): ENERGY STAR water heaters, US DOE.

## Air-conditioning electricity intensity (GJ per m² of cooled area per year)

| Vintage | QC | ON | AB | BC | CO | MI | NY | WA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pre-1960 | 0.053 | 0.043 | 0.009 | 0.029 | 0.042 | 0.043 | 0.063 | 0.050 |
| 1960–1983 | 0.041 | 0.033 | 0.007 | 0.023 | 0.029 | 0.036 | 0.060 | 0.033 |
| 1984–1999 | 0.029 | 0.023 | 0.005 | 0.016 | 0.027 | 0.020 | 0.041 | 0.022 |
| 2000+ | 0.024 | 0.019 | 0.004 | 0.013 | 0.024 | 0.018 | 0.029 | 0.019 |

Sources — Canada: NRCan CEUD Residential Table 4 (2018–2023 avg, shaped across vintage with the RECS gradient): QC · ON · AB · BC. US: EIA RECS 2020 microdata.

Cooling demand assumes the whole heated floor area is cooled.

## Other electricity — appliances, lighting, plug load (GJ per household per year), by household size

Other electricity = jurisdiction baseline (average household) × household-size factor.

Baseline — average household (GJ/hh):

| Parameter | QC | ON | AB | BC | CO | MI | NY | WA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | 20.2 | 12.8 | 16.2 | 17.0 | 21 | 21 | 16 | 20 |

Household-size factor (normalized so an average household = 1):

| Household size | 1 | 2 | 3 | 4 | 5 | 6+ |
| --- | --- | --- | --- | --- | --- | --- |
| Factor | 0.66 | 1.00 | 1.17 | 1.30 | 1.41 | 1.51 |

Sources — baseline: NRCan CEUD Residential Table 2, appliances + lighting per household (Canada); RECS 2020 non-HVAC electricity (US). Household-size factors: RECS 2020 (CO/MI/NY/WA), normalized to the size-weighted mean: EIA RECS 2020 microdata.

## Fuel emission factors (kg CO₂e per GJ, lifecycle)

Factors are lifecycle = combustion + upstream (well-to-tank). Upstream covers extraction, processing and transport of the fuel before it is burned (for natural gas, dominated by methane leakage).

| Fuel | Combustion | Upstream (WTT) | Lifecycle (used) |
| --- | --- | --- | --- |
| Natural gas | 50.3 | 12.98 | 63.3 |
| Heating oil / propane | 65.1 | 14.7 | 79.8 |
| Wood | 1.1 | — | 1.1 |

HHV basis, IPCC AR5 100-yr GWPs (CH₄ 28, N₂O 265). Combustion is EPA Hub 2025 (oil/propane = mean of distillate 70.3 & propane 59.8). Upstream adders come from the GREET-based DOE/RESNET set (oil +13.89, propane +15.54, mean +14.7) and are applied to both countries: ECCC does not publish a clean upstream-only residential value, and the Canadian regulatory full-lifecycle values agree within ~4%. Wood is biogenic-neutral; its harvesting/transport upstream is small and excluded for lack of a residential factor.

Sources — combustion: EPA GHG Emission Factors Hub 2025. Upstream (well-to-tank): DOE BTO Best Practices for Determining GHG Emissions Factors, Jan 2025, Table 2 and ANSI/RESNET/ICC 301-2022 Addendum B; cross-checked against Argonne GREET and the Canadian ECCC Fuel LCA Model / BC Reg. 295/2023

## Electricity grid emission factors (province/state-specific)

| Jurisdiction | g CO₂e/kWh | Grid loss | Combustion (delivered) | Upstream | Lifecycle (used) |
| --- | --- | --- | --- | --- | --- |
| QC | 1.7 | — | 0.47 | 0.2 | 0.7 |
| ON | 38 | — | 10.6 | 3.4 | 14.0 |
| AB | 490 | — | 136.1 | 22.7 | 158.8 |
| BC | 15 | — | 4.2 | 1.3 | 5.5 |
| CO | 495 | 4.1% | 143.4 | 16.1 | 159.5 |
| MI | 362 | 4.2% | 105.0 | 18.4 | 123.4 |
| NY | 212 | 4.2% | 61.5 | 14.7 | 76.2 |
| WA | 121 | 4.1% | 35.0 | 6.7 | 41.7 |

Combustion factors: Canadian are consumption-based (ECCC 2025; delivered basis incl. imports/exports, so no loss adjustment, shown “—”); US are eGRID2023 state output (busbar/generation) rates grossed up to a delivered basis by the eGRID subregion grid gross loss (CO via RMPA 4.1%, MI via RFCM 4.2%, NY 4.2%, WA via NWPP 4.1%). Upstream (precombustion) is the well-to-tank emissions of the fuels burned to generate, computed per grid as Σ(natural-gas/coal/oil generation share × heat rate × per-fuel upstream factor) expressed per delivered kWh — large on fossil grids (AB, CO, MI), near-zero on hydro/nuclear grids (QC, BC). Lifecycle = combustion + upstream (the value used). Grid factors are average, not marginal.

Sources — Canada combustion (consumption intensity 2025): ECCC, Emission Factors and Reference Values 2024, Table 5.2. US combustion (eGRID2023 output rates grossed up for grid loss): EPA eGRID2023 Summary Tables. Upstream/precombustion: computed from generation mix (eGRID2023 state resource mix; CER Provincial & Territorial Energy Profiles 2023) × per-fuel upstream (GREET/RESNET: natural gas 13.69, coal 7.0, oil 14.66 kg/MMBtu HHV).

## “Don’t know” option — country-average system mix

If a respondent does not know their space- or water-heating system, the calculator does not guess a single system — it substitutes the expected emissions across the country’s actual mix of systems. Because different systems burn different fuels at different efficiencies, the blend is taken over emissions, not over efficiency: for the selected jurisdiction,

expected emissions = useful demand × Σ (system share × fuel factor ÷ efficiency)

so the electric share is charged at that jurisdiction’s grid factor, the gas share at its furnace efficiency, and so on. Weights are national (one set for Canada, one for the US), since the system mix differs far more between the two countries than within them.

Space-heating system mix (% of homes):

| System | Canada | US |
| --- | --- | --- |
| Natural gas | 44 | 53 |
| Heating oil / propane | 6 | 9 |
| Electric baseboard / resistance | 37 | 22 |
| Electric heat pump | 8 | 15 |
| Wood | 5 | 2 |

Water-heating system mix (% of homes):

| System | Canada | US |
| --- | --- | --- |
| Natural gas | 49 | 48 |
| Electric resistance | 49 | 46 |
| Heating oil / propane | 2 | 6 |
| Heat-pump water heater | ~0 | ~0 |

(Shares may not sum to 100 due to rounding. Heat-pump water heaters are a negligible share of stock in both countries and are folded into electric resistance.)

Because the electric portion stays on each jurisdiction’s own grid, the “Don’t know” result falls below a gas home where the grid is clean (e.g. Ontario: 1,908 vs 3,165 kg for the worked-example house below) and above a gas home where the grid is dirty (e.g. Alberta).

Sources — system mix, Canada (national household stock, 2023): NRCan CEUD Table 27, Heating System Stock by Building Type and Heating System Type and Table 34, Water Heater Stock by Building Type and Energy Source. US: EIA RECS 2020 microdata.

## Consolidated sources for residential energy use

- NRCan Comprehensive Energy Use Database, Residential Sector (province Tables 2, 4, 21, 26, 33; national Tables 27 & 34 for the “Don’t know” system-stock mix): https://oee.nrcan.gc.ca/corporate/statistics/neud/dpa/menus/trends/comprehensive_tables/list.cfm

- EIA Residential Energy Consumption Survey (RECS) 2020 microdata: https://www.eia.gov/consumption/residential/data/2020/index.php?view=microdata

- EPA GHG Emission Factors Hub (2025): https://www.epa.gov/climateleadership/ghg-emission-factors-hub

- EPA eGRID2023: https://www.epa.gov/egrid

- Fuel & electricity upstream (well-to-tank): DOE BTO Best Practices 2025 / ANSI/RESNET/ICC 301-2022 Add. B: https://www.energy.gov/sites/default/files/2025-01/bto-ghg-metrics-011525.pdf

- Canada Energy Regulator, Provincial & Territorial Energy Profiles 2023 (grid generation mix): https://www.cer-rec.gc.ca/en/data-analysis/energy-markets/provincial-territorial-energy-profiles/provincial-territorial-energy-profiles-canada.html

- ECCC Emission Factors and Reference Values 2024 — Table 5.2, electricity consumption intensity for 2025 (Canadian grid factors): https://publications.gc.ca/collections/collection_2024/eccc/En84-294-2024-eng.pdf

- NRCan heat-pump performance: https://natural-resources.canada.ca/energy-efficiency/energy-star/heating-cooling-heat-pump

- NEEP cold-climate ASHP specification: https://neep.org/heating-electrification/ccashp-specification-product-list

- ENERGY STAR (furnaces, water heaters): https://www.energystar.gov/products/water_heaters

# Vehicle use

Vehicle emissions are taken from the Alternative Fuel Life-Cycle Environmental and Economic Transportation (AFLEET) online calculator: https://afleet.esia.anl.gov/afleet/payback-onroad-calculator, and converted from short tonnes per year assuming 12,400 miles driven annually to kg/km driven. Values for hybrid and electric vehicles are estimated by AFLEET per state based on emissions of the local electric grid. We map vehicle emissions for Canadian drivers to American states based on convex combinations of states with comparable electricity generation mix (based on the kg CO2e/GJ from EPA eGRID2023 and Canadian electricity consumption intensity values (Table 5.2) cited above. Using this strategy, Alberta is calibrated as a convex combination of Colorado and Michigan (0.981 / 0.019) and Ontario as a convex combination of Vermont and Washington (0.796 / 0.204), with weights chosen so the blended grid intensity (kg CO₂e/GJ) matches the province. Quebec and British Columbia have lower emissions per GJ than any US state, so for both we use the lowest-emission US grid, Vermont.

|  | kg CO₂e/GJ electricity | State 1 | State 2 | Share 1 | Share 2 | Convex combination of state emissions used |
| --- | --- | --- | --- | --- | --- | --- |
| BC | 5.5 | VT |  | 1 |  | 6.9 |
| AB | 158.8 | CO | MI | 0.981 | 0.019 | 158.8 |
| ON | 14.0 | VT | WA | 0.796 | 0.204 | 14.0 |
| QC | 0.7 | VT |  | 1 |  | 6.9 |

Driving emissions estimated in this way are as follows, and are divided by the typical number of people in the vehicle.

United States

| CO₂e kg/km | MI |  | CO |  | WA |  | NY |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fuel type | Truck | Car | Truck | Car | Truck | Car | Truck | Car |
| Gasoline | 0.315 | 0.215 | 0.315 | 0.215 | 0.315 | 0.215 | 0.315 | 0.215 |
| Diesel | 0.266 | 0.181 | 0.266 | 0.181 | 0.266 | 0.181 | 0.266 | 0.181 |
| HEV | 0.219 | 0.144 | 0.219 | 0.144 | 0.219 | 0.144 | 0.219 | 0.144 |
| PHEV | 0.140 | 0.098 | 0.123 | 0.086 | 0.123 | 0.086 | 0.116 | 0.081 |
| EV | 0.124 | 0.077 | 0.081 | 0.050 | 0.081 | 0.050 | 0.064 | 0.040 |
| Canada |  |  |  |  |  |  |  |  |
| CO₂e kg/km | BC |  | AB |  | ON |  | QC |  |
| Fuel type | Truck/SUV | Car | Truck/SUV | Car | Truck/SUV | Car | Truck/SUV | Car |
| Gasoline | 0.315 | 0.215 | 0.315 | 0.215 | 0.315 | 0.215 | 0.315 | 0.215 |
| Diesel | 0.266 | 0.181 | 0.266 | 0.181 | 0.266 | 0.181 | 0.266 | 0.181 |
| HEV | 0.219 | 0.144 | 0.219 | 0.144 | 0.219 | 0.144 | 0.219 | 0.144 |
| PHEV | 0.116 | 0.081 | 0.123 | 0.086 | 0.117 | 0.082 | 0.116 | 0.081 |
| EV | 0.064 | 0.040 | 0.082 | 0.051 | 0.067 | 0.042 | 0.064 | 0.040 |

# Current global average

Calculated personal emissions of survey respondents for diet, home heating, ground transport, and aviation are compared to estimated global per capita for these same categories.

| Category | t CO₂e pc/year | Source |
| --- | --- | --- |
| Diet | 2.0 | FAO, 2024 GHG emissions from agrifood systems: Global, regional and country trends, 2000-2022 |
| Home heating & cooling | 0.737 | 5.91 Gt 2002 (Our World in Data) / 8.02 billion people (Our World in Data) The IEA report on heating is found here: https://www.iea.org/energy-system/buildings/heating. The report on cooling is found here: https://www.iea.org/energy-system/buildings/space-cooling |
| Transport, including domestic aviation | 1.02 | Climate Watch (2026), via Our World in Data, 2023 (Excludes international aviation) |
| International aviation | 0.062 | OECD (2025); Population based on various sources (2024) – with minor processing by Our World in Data, via Our World in Data, 2024 |
| Total | 3.819 |  |

Note: Total GHG emissions per capita (including the categories captured above, plus others such as waste, oil fuel extraction, industrial processes) were 6.56 CO2e globally in 2024, as reported by the Emissions Database for Global Atmospheric Research: https://edgar.jrc.ec.europa.eu/report_2025

# Global target per capita

The global per capita target of 2.5 is based on analysis of “lifestyle” carbon emissions consistent with keeping the probability of exceeding 1.5°C average global temperature increase below 50%, as estimated by the Hot or Cool Institute (https://hotorcool.org/publications/1-5-degree-lifestyles-towards-a-fair-consumption-space-for-all/) and promoted by Scientists for Global Responsibility (https://www.sgr.org.uk/projects/fair-lifestyle-targets-additional).

Appendix

Table 1. Popular Flight Destinations by EPA Distance Category

| City/Airport | Destination | Distance (miles) |
| --- | --- | --- |
| New York City (JFK/LGA/EWR) |  |  |
| SHORT-HAUL (<300 miles) |  |  |
|  | Boston | 190 |
|  | Philadelphia | 95 |
|  | Baltimore | 170 |
|  | Washington DC | 225 |
|  | Buffalo | 290 |
| MEDIUM-HAUL (300-2,300 miles) |  |  |
|  | Chicago | 710 |
|  | Atlanta | 760 |
|  | Orlando | 940 |
|  | Miami/Fort Lauderdale | 1,100 |
|  | Dallas | 1,370 |
| LONG-HAUL (>2,300 miles) |  |  |
|  | Los Angeles | 2,450 |
|  | San Francisco | 2,570 |
|  | London | 3,450 |
|  | Paris | 3,625 |
|  | Rome | 4,280 |
| Toronto (YYZ) |  |  |
| SHORT-HAUL (<300 miles) |  |  |
|  | Buffalo | 60 |
|  | Ottawa | 220 |
|  | Detroit | 230 |
|  | Pittsburgh | 210 |
| MEDIUM-HAUL (300-2,300 miles) |  |  |
|  | Montreal | 315 |
|  | New York/LaGuardia | 340 |
|  | Miami | 1,200 |
|  | Fort Lauderdale | 1,200 |
|  | Calgary | 1,675 |
| LONG-HAUL (>2,300 miles) |  |  |
|  | London | 3,550 |
|  | Paris | 3,730 |
|  | Frankfurt | 3,850 |
|  | Manila | 8,206 |
| Montreal (YUL) |  |  |
| SHORT-HAUL (<300 miles) |  |  |
|  | Ottawa | 110 |
|  | Burlington | 80 |
|  | Boston | 250 |
|  | Quebec City | 155 |
| MEDIUM-HAUL (300-2,300 miles) |  |  |
|  | Vancouver | 2,300 |
|  | Chicago | 745 |
|  | Miami | 1,330 |
|  | Calgary | 1,900 |
|  | Toronto | 315 |
| LONG-HAUL (>2,300 miles) |  |  |
|  | London | 3,250 |
|  | Paris | 3,425 |
|  | Frankfurt | 3,730 |
|  | Casablanca | 3,450 |
| Seattle (SEA) |  |  |
| SHORT-HAUL (<300 miles) |  |  |
|  | Portland | 130 |
|  | Vancouver BC | 125 |
|  | Spokane | 225 |
|  | Kelowna | 240 |
| MEDIUM-HAUL (300-2,300 miles) |  |  |
|  | San Francisco | 680 |
|  | Los Angeles | 960 |
|  | Las Vegas | 870 |
|  | Phoenix | 1,110 |
|  | Denver | 1,020 |
| LONG-HAUL (>2,300 miles) |  |  |
|  | Honolulu | 2,675 |
|  | Tokyo | 4,790 |
|  | London | 4,785 |
|  | Frankfurt | 5,070 |
|  | Paris | 4,950 |
| Detroit (DTW) |  |  |
| SHORT-HAUL (<300 miles) |  |  |
|  | Chicago | 235 |
|  | Toronto | 230 |
|  | Cleveland | 90 |
|  | Pittsburgh | 205 |
|  | Cincinnati | 230 |
| MEDIUM-HAUL (300-2,300 miles) |  |  |
|  | Los Angeles | 1,980 |
|  | Orlando | 985 |
|  | Las Vegas | 1,740 |
|  | Denver | 1,120 |
|  | Dallas | 985 |
| LONG-HAUL (>2,300 miles) |  |  |
|  | Amsterdam | 3,860 |
|  | London | 3,740 |
|  | Shanghai | 7,118 |
| Vancouver (YVR) |  |  |
| SHORT-HAUL (<300 miles) |  |  |
|  | Victoria | 65 |
|  | Seattle | 125 |
|  | Portland | 250 |
|  | Kelowna | 190 |
|  | Nanaimo | 50 |
| MEDIUM-HAUL (300-2,300 miles) |  |  |
|  | Toronto | 2,100 |
|  | San Francisco | 810 |
|  | Los Angeles | 1,080 |
|  | Las Vegas | 850 |
|  | Phoenix | 1,250 |
| LONG-HAUL (>2,300 miles) |  |  |
|  | Manila | 6,700 |
|  | Delhi | 7,000 |
|  | Bangkok | 7,600 |
|  | Hong Kong | 6,380 |

Anthropic. (2026). Claude 4.5 (Sonnet version) [Large language model]. https://claude.ai