# Heat-pump backup model — decisions & parameters (2026-07-20)

Final model added to CO2_calculator_all_jurisdictions.xlsx for heat pumps with a backup heat source.

## Model

- COP(T) curve: field data, Gibb et al. 2023 (Joule) LOWESS trend, digitized → COP_field(T) = 2.827 + 0.0807·T + 0.00151·T² (RMSE 0.026). Chosen over datasheet curves (Staffell 2012, Ruhnau 2019); it is standard-stock field data, NOT cold-climate (ccASHP) — Gibb treats ccASHP separately and that data was excluded.

- Lockout: −15 °C (standard-stock compressor cutout; resistance/backup below).

- Climate: seasonal sinusoid + N(σ=4°C) daily variability; heating load ∝ max(0,18−T).

- Level calibration: single factor φ scales the curve so national load-weighted seasonal COP = 1.90 (CA) / 2.00 (US). φ_CA=0.731, φ_US=0.626; ratio 0.855 → US fleet ~14% worse at equal climate (vintage). Climate handled explicitly per jurisdiction.

- Electric/None/Don't-know backup: jurisdiction-specific seasonal COP (integrate φ·curve over each jurisdiction's climate). The single seasonal COP is the exact (lossless) load-weighted harmonic mean of the curve — verified identical to bin-by-bin integration.

- Fossil (gas, oil/propane) backup — switchover: economic (cost-minimizing) temperature, COP* = (elec$/kWh × η)/(fuel$/GJ × 0.0036), solved on φ·curve. Capped at max 0 °C (HP-priority above freezing) and floored at −15 °C. User decision: pure economics put Ontario switchover at +15°C (HP idle all winter) because gas is cheap; the 0°C cap fixes this. Cheap-electricity provinces (QC/MB/BC) run HP to lockout via the economic value.

- Emissions (fossil backup): E = f_hp·(D/COP_op)·g + (1−f_hp)·(D/η_b)·e_b. Lifecycle EFs: gas 63.28, oil/propane 79.8 kg CO2e/GJ; grid factor per jurisdiction.

## Workbook changes

- New input B15 "Heat-pump backup heat source" (dropdown). All Calculator rows ≥15 shifted +1 (space heating now row 26, TOTAL row 30).

- P_HPcop repurposed: B=jurisdiction seasonal COP, C/D=fhp/COP gas, E/F=fhp/COP oil-propane, G/H=switchover temps, I/J=elec & gas prices. (It's the 14th of 16 tabs — easy to miss.)

- Auditable HP-backup detail block at Calculator K24:L32.

- Space-heating formula B26 gets a heat-pump branch → uses L32. Non-HP unchanged; Don't-know-mix now inherits jurisdiction HP COP (intended).

## Jurisdiction set — NOW 66 (PR + VI added 2026-07-20)

Originally 64 (50 US states + DC + 13 CA). Puerto Rico and US Virgin Islands added as full jurisdictions across all 10 keyed sheets + P_Vehicle + the B5 hint list. Both tropical → space heating ≈ 0, HP-backup path inert; AC + other-elec dominate. PR grid EF = eGRID2022 PRMS ≈726 gCO2e/kWh; VI (WAPA oil) estimated higher. Non-electricity params (heating/cooling/water/housing) set from a Hawaii analogue with tropical adjustments. Verified: PR/VI produce valid footprints, no #N/A; existing jurisdictions byte-identical.

## Deliverables (in "Claude output" folder)

- CO2_calculator_all_jurisdictions.xlsx — the calculator.

- Heat pump backup methodology.docx — methodology writeup.

- Calculator implementation spec (for JavaScript).docx — tab-by-tab + formula-by-formula spec with a reference JS footprint() function, for porting the calculator to JavaScript in another session.

## Prices (2024–26)

US: EIA/ChooseEnergy state electricity (¢/kWh) & gas (/Mcf→/Mcf→ /Mcf→/GJ). Canada: energyhub provincial electricity; per-province delivered gas estimates ($9–16/GJ). Oil/propane ~$28/GJ US, ~$40/GJ CA. PR elec 25¢, VI 45¢.

## Verified

LibreOffice recompute throughout: non-HP systems byte-identical to original; HP-electric uses jurisdiction SPF; None/Don't-know backup = electric; dual-fuel sensible; PR/VI valid.